module vending_machine(clk, rst, x, out, change);

input clk, rst;

input [1:0] x;

output reg out;

output reg [1:0] change;

reg [1:0] p_state, n_state;

parameter S0 = 2'b00;

parameter S1 = 2'b01;

parameter S2 = 2'b10;

always @(p_state or x)

begin

n_state = S0;

change= 2'b00;

case(p_state)

S0: if(x == 2'b01) begin 				//Received 5 Rs

n_state = S1;

change = 2'b00;

out = 1'b0;

end

else if(x == 2'b10) begin  		//Received 10 Rs

n_state = S2;

change = 2'b00;

out = 1'b0;

end

else begin

n_state = S0;

change = 2'b00;

out = 1'b0;

end

S1: if(x == 2'b01) begin 				//Received 10(5+ 5) Rs

n_state = S2;

change = 2'b00;

out = 1'b0;

end

else if(x == 2'b10) begin 		             //Received 15(5+10) Rs

n_state = S0;

out = 1'b1;

change = 2'b00;

end

else begin

n_state = S0;

change = 2'b01;

out = 1'b0;

end

S2: if(x == 2'b01) begin 					//Received 15(5+ 5 + 5) Rs

n_state = S0;

out = 1'b1;

change = 2'b00;

end

else if(x == 2'b10) begin 				 //Received 20(10+ 10) Rs

n_state = S0;

out = 1'b1;

change = 2'b01;

end

else begin

n_state = S0;

change = 2'b10;

out = 1'b0;

end

endcase

end

always @(posedge clk or posedge rst)

if(rst)

p_state <= S0;

else

p_state <= n_state;

endmodule